Enzymatic Assay of NUCLEOSIDE PHOSPHORYLASE

(EC 2.4.2.1)

PRINCIPLE:

Inosine + Pi Hypoxanthine + Ribose-1-PO4

Hypoxanthine + 2H2O + 2O2 Uric Acid + 2H2O2

Abbreviations:
Pi = Inorganic Phosphate

CONDITIONS: T = 25°C, pH = 7,4, A293nm, Light path = 1 cm

METHOD: Continuous Spectrophotometric Rate Determination

REAGENTS:

- 100 mM Potassium Phosphate Buffer, pH 7,4 at 25°C
(Prepare 100 ml in deionized water using Potassium Phosphate, Monobasic.) Adjust to pH 7,4 at 25°C with 1 M NaOH.)

- 7,5 mM Inosine Solution
(Prepare 5 ml in deionized water using Inosine)

- Xanthine Oxidase Enzyme Solution
(Immediately before use, prepare a solution containing
10 units/ml of Xanthine Oxidase, in ice cold deionized water.)

- Nucleoside Phosphorylase Enzyme Solution
(Immediately before use, prepare a solution containing 0,125 units/ml of Nucleoside
Phosphorylase in ice cold deionized water.)

PROCEDURE:

Pipette (in milliliters) the following reagents into suitable cuvettes:

|  | Test | Blank |
| --- | --- | --- |
| Reagent A (Buffer) | 2,70 | 2,70 |
| Reagent B (Inosine) | 1,0 | 1,0 |
| Regent C (Xanthine Oxidase) | 0,10 | 0,10 |

Mix by inversion and equilibrate to 25°C. Monitor the A293nm until constant, using a suitably

thermostatted spectrophotometer. Then add:

| Reagent d (Nucleoside Phosphorylase) | 0,10 | ------ |
| --- | --- | --- |
| Deionized Water | ------ | 0,10 |

Immediately mix by inversion and record the increase in A293nm for approximately 5 minutes. Obtain the ΔA293nm/minute using the maximum linear rate for both the Test and Blank.

CALCULATIONS:

Units/mg enzyme = (ΔA293nm/min Test - ΔA293nm/min Blank)(12.0)(mg enzyme/ml RM)

12,0 = Millimolar extinction coefficient of Uric Acid at 293nm

RM = Reaction Mix

UNIT DEFINITION:

One unit will cause the phosphorolysis of 1,0 μmole of inosine to hypoxanthine and ribose
1-phosphate per minute at pH 7,4 at 25°C.

FINAL ASSAY CONCENTRATION:

In a 3,0 ml reaction mix, the final concentrations are 90 mM potassium phosphate, 0,25 mM inosine, 1,0 units of xanthine oxidase and 0,0125 units of nucleoside phosphorylase.

NOTES:

1. Xanthine Oxidase - One unit will convert 1,0 μmole of xanthine to uric acid per minute at pH 7,5 at 25°C.